Performance Study of the MPI and MPI-CH Communication Libraries on the IBM SP

Ewa Deelman and Rajive Bagrodia

UCLA Computer Science Department

deelman@cs.ucla.edu, rajive@cs.ucla.edu

http://pcl.cs.ucla.edu

During our research in the simulation of message-passing applications on parallel, high-performance systems, we have discovered that the native MPI implementation on the IBM SP suffers from performance anomalies. Our simulator, MPI-Sim, predicted a smooth performance for an ASCI relevant application, whereas the measured system showed a sudden jump in the runtime of the application. Surprisingly, the MPI-CH implementation on the same machine does not suffer from the same performance degradation.   This report identifies the anomaly and summarizes the relative performance results for MPI and MPI-CH for SWEEP3D, a large scientific application.

In our research, we have developed a simulator, MPI-Sim[1-3], which simulates large-scale applications written with MPI. Currently, MPI-Sim can simulate the communication library on the IBM SP and the Origin 2000.  MPI-Sim uses direct execution for the sequential portions of the code. The MPI calls are trapped by the simulator and their behavior is modeled in detail.

Under the DARPA funded POEMS project[4], we have been studying the performance of an ASCI kernel applicationSweep3D[5] on the IBM SP. The benchmark code SWEEP3D represents the heart of a real ASCI application. It solves a 1-group time-independent discrete ordinates 3D Cartesian geometry neutron transport problem. SWEEP3D exploits parallelism via a wavefront process. First, a 2 dimensional spatial domain decomposition onto a 2D array of processors in the I-and J-directions is used. A single wavefront solve on these domains provides limited parallelism. To improve parallel efficiency, blocks of work are pipelined through the domains. SWEEP3D is coded to pipeline blocks of mk k-planes and mmi angles through the 2D processor array. The original application was written in Fortran, however in order to use our simulator, we translated the code to C using f2c.

The machine we are using is the Blue system at Lawrence Livermore Laboratory. The system currently has 158 nodes each with four 332 MHz 604e processors, sharing 512 MB of memory and attached to 1GB disks. The inter-node communications of the SP give a bandwidth of 100 MB/second and a latency of 35 microseconds with the use of SP High Performance Switch TB3. In our experiments we have targeted the MPI communication library provided by IBM as our modeling object.. We have used MPI-Sim to study the scalability of MPI applications; in particular to investigate the impact of adding additional processors to the execution time of the program. Two primary problem configurations were used: first, where the total problem size under consideration remained constant as more processors were added and second, where the problem size per processor was kept constant, so the total problem size increased as more processors were added. The latter experiment was used to estimate the performance of the application on thousands of processors.

To determine the accuracy of the simulator, we have compared the runtime predicted by the MPI-SIM simulator to the measured runtime of the application. For the constant total problem size, the simulator accurately predicted the runtime (to within 5%see Figures 1 and 2.)

However, the second set of experiments, which were designed to predict the performance of 20 million and one billion total problem sizes (2553 and 10003 cells) on thousands of processors showed a discrepancy between the predicted behavior and the measured system. The first step in the study was to decompose the problem into a number of homogenous grids such that each grid can be mapped to a unique processor.  As the problem must be mapped onto a 2-D processor grid, the size of the third dimension in each grid is fixed by the shape of the original problem.  Thus, for a problem size of 20 million and 20,000 processors, the size per processor is about ((2*107/2*104)=103). As the size of the k dimension must be 255, the resulting shape of the per processor grid is 22255. For the 20 million problem, we looked at per processor grid sizes of 22255, 44255 and 77255 based on a selection of processor configurations of 20,000, 4,900 and 1,600 processors respectively. For the billion size problem, the mapping to a machine with 20,000 processors yielded a per processor grid size of 661000.  In all experiments, only one processor of the 4-way SMP node was used, which allowed for the fast user space communications.

For all the problem sizes, we noticed a discrepancy between our model and the measured system performance (see Figure 3). The MPI-Sim model shows a smooth increase in execution time as the number of processors (and corresponding problem size) is increased. The measured system shows a sudden increase in execution time.  The specific machine sizes at which the anomaly was observed appeared to depend on the problem size under investigation: from 64 to 81 processors for the smallest size, 25 to 36 for the 44255 grid size, and 16 to 25 for the largest size.  A similar performance study was performed by researchers at University if Wisconsin [6]. The analytical LogGP models presented in that work also predicted smooth performance rather than the performance jump observed in the measured system behavior (see Figure 4.)  The figure shows that the measured system performance for the 661000 per processors size is smooth for up to 36 processors, having a runtime of approximately 29 seconds. Then, the performance decreases (at 36 processors) as the runtime increases from 29.1 to 36.1 seconds. For more than 36 processors, the performance is again smooth.

Substantial effort was devoted to trying to find the cause of the performance discrepancy between the measured system and the simulation results, but none of the alternatives appeared  to explain the discrepancy satisfactorily. Since the problem size per processor remains constant as the number of processors is increased, cache effects do not play a role. Also, the size of messages sent does not change as the number of processors is increased. Eventually, the UCLA and Wisconsin researchers were forced to publish the results leaving the cause of discrepancy as an open question for the community[1, 6].  Based on subsequent experiments with other MPI implementations as described in this paper, we now believe that the performance anomalies were due to anomalies in the specific implementation of the collective communication operations in the library.

The experiments described previously were conducted in October 1998, since then the Blue machine at LLNL was upgraded. In July of 1999 some of the experiments were rerun. This time, measurements were taken using the MPI implementation provided by IBM (henceforth referred to as MPI-IBM) with the MPI-CH

implementation that was also available on the machine. Again, we looked at Sweep3D with different per processor problem sizes.  For the IBM MPI version, the code was compiled with the mpcc script. The MPI library was located in /usr/lpp/ppe.poe/lib.The MPI-CH (version 1.1.1 located in /usr/local/mpi/lib/rs6000/ch_mpl) code is compiled with the mpicc script . Both mpcc and mpicc call the IBM xlc compiler for compilation. The compilation options used in both cases had options: -O3 -qstrict. The results are depicted in Figures 5 and 6.  The MPI-IBM still has the performance degradation, although in different locations than in previous experiments. Surprisingly, MPI-CH has a smooth performance curve, it also outperforms MPI-IBM in many processor configurations. Additionally, we have tuned MPI-Sim to model the MPI-CH communication library (MPICH-SIM in the graphs).  We can see (Figures 5 and 6) that MPICH-SIM can accurately predict the performance of MPI-CH. The results show that MPI-Sim can accurately capture the behavior of a message-passing library on the IBM SP.  Furthermore, it appears that the MPI-IBM is adapting its protocols based on the number of communicating processors, or number of messages in the system (since the message size nor the number of messages sent by a process are changed as the number of processors is increased).  In this case the protocol changes result in poor performance.  If the protocols were kept constant, the application's behavior would have been smooth, as predicted by MPI-Sim.

All of the above experiments were conducted based on the version of Sweep3D translated to C from Fortran. For completeness, we compared the measured execution time of the Fortran version of the code and the 661000 per processor grid size. The results (see Figure 7) are similar to the C code performance. MPI-IBM implementation shows a sudden degradation in system performance when the number of processors is increased from 25 to 36. The magnitude of the jump is 37%. However, unlike the C version of MPI-IBM outperforms MPI-CH for a small number of processors (4 to 25).

Conclusion

We have studied the performance of the MPI communication library provided by IBM and MPI-CH on the newest generation IBM SP. The high-performance user space communications were used. We based our experiments on the Sweep3D application, were the problem size per processor was kept constant as the number of processors was increased. We have found that the IBM’s MPI suffers from sudden performance degradation.  We were able to determine that the problem lies in the MPI implementation, since MPI-CH does not exhibit this behavior. Based on the application behavior, we suppose that the problem lies in the collective MPI communications. Additionally, for this application, MPI-CH has superior performance in most cases.

Acknowledgements

This work was supported by the Advanced Research Projects Agency DARPA/ITO under Contract N66001-97-C-8533, “End-to-End Performance Modeling of Large Heterogeneous Adaptive Parallel/Distributed Computer/Communication Systems.”

References

1.	Bagrodia, R., et al. Performance Prediction of Large Parallel Applications using Parallel Simulations. in 7th ACM SIGPLAN Symp. on Principles and Practice of Parallel Programming. 1999. Atlanta, GA.

2.	Prakash, S. and R.L. Bagrodia. MPI-SIM: using parallel simulation to evaluate MPI programs. in Proceedings (Cat. No.98CH36274) Proceedings of IEEE Winter Simulation Conference. 1998. Washington, DC, USA: IEEE.

3.	Deelman, E., et al. POEMS: End-to-end Performance Design of Large Parallel Adaptive Computational Systems. in First International Workshop on Software and Performance. 1998. Santa Fe, NM.

4.	The ASCI Sweep3d Benchmark Code. http://www.llnl.gov/asci_benchmarks/

5.	Sundaram-Stukel, D. and M.K. Vernon. Predictive Analysis of a Wavefront Application using LogGP. in 7th ACM SIGPLAN Symp. on Principles and Practice of Parallel Programming. 1999. Atlanta, GA.

Figure 1: Validation of MPI-SIM Predicting the Performance of Sweep3D on the LLNL IBM SP. The Total Problem Size is Constant (1503).

Figure 2: Validation of MPI-SIM with 503 Total Problem Size.

(a) 22255 Per Processor Problem Size, the Jump Occurs between 64 and 81 processors; the runtime jumps from 1.74 to 2.87 seconds.

(b) 44255 Per Processor Problem Size, the Jump Occurs between 25 and 36 processors; the runtime jumps from 3.97 to 5.11 seconds.

(c) 77255 Per Processor Problem Size, the Jump Occurs between 16 and 25 processors; the runtime jumps from 9.57 to 11 seconds.

Figure 3:  Comparison Between the Measured System and MPI-SIM Predictions.

Measurements were performed in October 1998.

Figure 4:  66x1000 Per Processor Problem Size. The Measured System is Compared to Simulation (MPI-Sim) and Analytical Models (LogGP).

Figure 5: Performance Comparison Between MPI and MPICH for 661000 Per Processor Problem Size.

(a) 22255 Per Processor Problem Size, the Jump Occurs between 64 and 81 processors.

(b) 44255 Per Processor Problem Size, the Jump Occurs between 64 and 81 processors.

(c) 77255 Per Processor Problem Size, the Jump Occurs between 16 and 25 processors.

Figure 6:  Performance Comparison Between MPI and MPI-CH.

Figure 7: Performance Comparison Between MPI and MPICH for the Fortran Sweep3D code with a Constant 661000 Per Processor Problem Size.